Organization Background

Name: City Health Hospital

City Health Hospital is a mid-sized healthcare facility offering comprehensive medical services, including outpatient consultations, inpatient care, diagnostic services, and administrative support. The hospital operates out of a single building with five distinct departments:

- Administration

- Emergency/Outpatient Department (OPD)

- Inpatient/ICU Department

- Laboratory and Radiology

- Server (for appointment scheduling, patient tracking)

Current State

The hospital’s existing network infrastructure struggles with poor connectivity, limited scalability, and inadequate security measures. This has resulted in inefficiencies in patient care, delays in communication, and risks to sensitive medical data. A redesigned LAN is essential to address these challenges.

Benefits of the Proposed Network Design

The proposed LAN design offers:

- Improved Performance: Network segmentation reduces congestion.

- Scalability: Supports future expansion of devices and departments.

- Enhanced Security: Subnetting and firewalls protect sensitive medical and administrative data.

- Centralized Management: Simplified troubleshooting and maintenance.

- Cost Savings: Efficient resource allocation and energy-saving hardware.

- Wireless Connectivity: Improved mobility for healthcare staff in the Emergency/OPD department.

Network Design Overview

Logical Network Diagram

(Design created using Cisco Packet Tracer)

Subnet Structure

- Administration (Class A): 10.0.0.0/24

- Emergency/Outpatient Department (OPD) (Class A): 10.0.1.0/24

- Inpatient/ICU Department (Class B): 172.16.0.0/24

- Laboratory and Radiology (Class C with Wireless LAN): 192.168.1.0/24

- Server Segment: Isolated network segment for servers.

Devices per Department

| Department | Devices |
| --- | --- |
| Administration | 3 Workstations, 1 Printer, 1 IP Phone |
| Emergency/OPD | 3 Workstations, 1 Tablet, 1 IP Phone |
| Inpatient/ICU | 3 Workstations, 1 Laptop, 1 IP Phone |
| Laboratory and Radiology | 2 Workstations, 2 Tablets, 1 Wireless Printer |
| Server Segment | Appointment Server, Patient Tracking Server |

Network Configuration

Router Configuration

The router acts as a DHCP server to assign IP addresses dynamically to all nodes except servers.

DHCP Configuration (Sample Code):

Router(config)# ip dhcp pool ADMIN

Router(dhcp-config)# network 10.0.0.0 255.255.255.0

Router(dhcp-config)# default-router 10.0.0.1

Router(dhcp-config)# dns-server 10.0.0.10

Router(config)# ip dhcp pool OPD

Router(dhcp-config)# network 10.0.1.0 255.255.255.0

Router(dhcp-config)# default-router 10.0.1.1

Router(dhcp-config)# dns-server 10.0.1.10

Router(config)# ip dhcp pool ICU

Router(dhcp-config)# network 172.16.0.0 255.255.255.0

Router(dhcp-config)# default-router 172.16.0.1

Router(dhcp-config)# dns-server 172.16.0.10

Router(config)# ip dhcp pool LAB

Router(dhcp-config)# network 192.168.1.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.1.1

Router(dhcp-config)# dns-server 192.168.1.10

VLAN Configuration

Each department connects to the main router via distribution switches with VLANs:

- VLAN 10: Administration

- VLAN 20: Emergency/OPD

- VLAN 30: Inpatient/ICU

- VLAN 40: Laboratory and Radiology

- VLAN 50: Server Segment

VLAN Commands (Sample):

Switch(config)# vlan 10

Switch(config-vlan)# name ADMIN

Switch(config)# vlan 20

Switch(config-vlan)# name OPD

Switch(config)# vlan 30

Switch(config-vlan)# name ICU

Switch(config)# vlan 40

Switch(config-vlan)# name LAB

Switch(config)# vlan 50

Switch(config-vlan)# name SERVER

Wireless LAN (Laboratory and Radiology Department)

The Laboratory and Radiology department uses a wireless LAN to support mobility. A wireless access point (AP) is configured for this subnet.

Access Point Configuration:

- SSID: CityHealthLab

- Authentication: WPA2-Enterprise

- Frequency: 2.4 GHz and 5 GHz

Network Security

Measures Implemented:

- Firewall: Configured on the router to block unauthorized access.

- Access Control Lists (ACLs): Restrict access between VLANs.

- Encryption: WPA2 for wireless communication.

- Server Isolation: Servers in the server segment are accessible only via specific VLANs.

- Regular Updates: Software updates for all devices to patch vulnerabilities.

ACL Configuration Example:

Router(config)# access-list 100 permit ip 10.0.0.0 0.0.255.255 172.16.0.0 0.0.0.255

Router(config)# access-list 100 deny ip any any

Router(config-if)# ip access-group 100 in

Server Configuration

Servers are configured in the Server Segment as follows:

| Server | Function | IP Address |
| --- | --- | --- |
| Appointment Server | Handles scheduling | 10.0.2.1 |
| Patient Tracking Server | Tracks patient records | 10.0.2.2 |

Conclusion

The proposed LAN design will provide City Health Hospital with a reliable, secure, and scalable network. By leveraging VLANs, DHCP, and enhanced security measures, the network will meet current operational needs and support future growth.

| Devices | User executive password | Privilage executive password |
| --- | --- | --- |
| Router 1 | 7413 | 2080 |
| —------- |  |  |
| Multi Switch 1 | time | time1 |
| —-------- |  |  |
| Switch 1 | yeka | yeka1 |
| Switch 2 | yeka | yeka1 |
| Switch 3 | yeka | yeka1 |
| Switch 4 | yeka | yeka1 |
| Switch 5 | yeka | yeka1 |
| Switch 6 | yeka | yeka1 |